Career Episode 1

Productivity development project of the fluorescent frame of household and office consumes

- Introduction

[CE 1.1] This project started at the beginning of 2015 in the company … located in Iran, Tehran, Eshtehard industrial park and It lasted for 6 months, with the aim of improving the production processes of lighting industry products and restudying the design of work flows and optimizing production processes and increasing the productivity and quality of products. I worked full-time in this project as an industrial engineer in the planning unit. I also worked in this company for 1 year.

- Background

[CE 1.2] Nature of Overall Engineering Project

Factory was established in 2013 and has started its activity in the field of producing all kinds of lighting lamps. In 2015, this factory started its activity in the field of electronic products. And then it was bought by the group in 2016 to help its development and prosperity. This factory, with an area of 6000 square meters, with more than 170 official, technical and production personnel, has produced more than 270 items of products, including frames (Household, office, industrial, park/lawn, etc.). The scope of my duties in this factory has been on the production line of household and office products and includes various departments, including: Warehouse of primary and final products - metal working unit - washing and painting unit - plastic injection unit - assembly unit - drawing and molding unit - quality control unit.

The aim of this factory has always been superior quality - greater product variety and more importantly, the satisfaction of respected consumers. And according to the prospects of the group, with the aim of increasing the quality of production, it tried to take a significant share in the market, and in the long term, it considered exporting to neighboring countries such as Iraq and Afghanistan.

[CE 1.3] Objectives of Project

Due to the fact that the documentation of processes, industrial design and layout of machines, proper planning in the allocation of labor to machines, attention to the correct recording of information, attention to the basic processes of quality control, were not done according to the standard framework, the factory always has a crisis of low quality, and has faced high waste products as well as low sales and staff dissatisfaction. Process of creating the fluorescent frames is almost the same for all kind of consumers. I will examine it further to show how to improve the production process of fluorescent frames. Therefore, the goals of the project include the following:

  - Restudy in the layout of the devices for better order and reducing the time of transferring goods between the devices, which led to better management and reduction of temporary warehouses.

  - Identifying and documenting the various processes involved in production, which led to a correct understanding of the activities.

  - Identifying production challenges and bottlenecks and evaluating the work and time of technical personnel, which led to the correct allocation of labor and ultimately increased production.

  - Improving quality control processes that had a significant role in reducing wastes.

[CE 1.4] Nature of Particular Work Area

The environment, in which I was engaged in design and optimization, was interactive and I collaborated with a group of people with electrical engineering expertise as well as technical technicians in the field of working with various production devices. Also, the whole process of purchasing the primary items to assembly and selling the products was done in especially in the current space of the factory. It can be said that despite all these people there, it was possible to evaluate the impact of the measures taken.

[CE 1.5] Hierarchy

[CE 1.6] Duties

• I prepared all work and production processes in the form of documents and visuals.

• All the times that were done for the production of parts by the workforce were identified and by considering the number of production required per day and the standard time of the machine and the number of personnel, I proposed an optimal scheme.

• Due to the lack of sufficient data, which was caused by not recording the information correctly, I made all the units responsible for recording the information correctly in the worksheets and in the specified Excel files, so that proper reports can be created as a result.

• By changing the process of moving semi-finished products between machines, I minimized the number of temporary warehouses, which led to the creation of more open space between machines.

• I restudy the quality control processes of the products. I added the process of quality control during production and quality assurance at the end of production to the cycle, which had a great impact.

• I made suggestions regarding the automation of some parts in order to eliminate manpower.

- Personal Engineering Activity

[CE 1.7] Technical Details of Work

In the first step, I got to know the production space and its various devices and information. For the technical study of this product, I prepared documents that specifically include information on raw materials, parts to be purchased, parts to be built, available machines, and the capacity of each machine, considering its maintenance and repairs time spans and the number of assigned personnel.

Next, I prepared operational sheets that display the combination of the above information in full detail so that the production process can be specified in it. I also drew diagrams to document production operations (OPC) and work flows (FPC), as well as product assembly and disassembly processes, which led to an overview and sufficient understanding.

Also, I determined the daily and seasonal production plans and created forms for the demand and inventory of finished products and raw materials to create a new procedure in the factory and create a better relationship between warehouse, production line and sale departments.

I also periodically checked the quality of semi-finished and purchased products and this sensitivity improved the situation. For example, in the operation of punching cut sheets and forming them, the two side holes created in the sheet should be placed in the bent part so that we do not face problems in the next steps, especially assembly. Of course, the distance of the two side holes from the edge of the sheets should be equal; otherwise, up to 2 mm error can be ignored. These things were noticeable at first. But by evaluating the correct performance of the personnel and checking the devices and by measures such as stopping the production line, the wastes were significantly reduced.

(The image below shows the status of a sheet that was rejected in the quality control process)

I was in charge of controlling the process of painting the final products. Because there were defects observed, within both the product that was ready for assembly and the product that was provided to the customer. Such as the lack of uniformity of paint spraying, the corrosion of some painted parts over time. Finally, with a lot of technical reviews and research, because all the painting and washing steps were done by technical personnel, I suggested creating a fully automatic paint spraying unit with two paint spraying robots. This action had an incredible effect on the color quality of the final products.

[CE 1.8] How you applied your Engineering Knowledge and Skills

According to the capacity of the machines, the production plan was approved for 250 frames per day, 150 units of office products and 100 units of home products. But in most cases, the total production per day did not reach more than 170 final bodies.

At first, for each operation that was performed on a machine, a work component registration sheet was designed and documented to record all the activities performed by a person. Also, the allowed idle time sheet for each machine was also included. In the following, all time measurement observations during work were also recorded for the intended workers and in different cycles (22 cycles for each operation) to get an average production time for each product. (In different cycles, we see different events, including the technical check of the machine - product quality control - shift change - talking to others, etc., each of which has a specific effect on the overall schedule.) And then by calculating the average normal operation time and the percentage of idle time, the standard time of each operation was calculated.

Standard Time = Normal Time + (Normal Time * percentages of idle Time)

By designing precedence diagram for the production processes of the intended products, which specifies the number of machines, the calculated standard time of each operation and the synchronicities, an analysis was done on the person-machine and overall execution times.

In total, during the day, 18 people are working on the devices in the production line until the final assembly, and according to the review of the operations that are in the sequence, we need to allocate manpower to each operation.

Cutting machines-form 1-form 2- punch- blade- sword-punching with coil are each located in a separate department and they all need workers for production, so we must assign one worker to each of them.

But in the injection department, due to the existence of two injection machines in this department, and due to the fact that in these two machines, we have to make the belt- Screws and plastic pieces, and due to the automaticity of the injection machine, a reduce can occur in the number of workers in this department. Despite some operation times are less than other operations, the task of the worker related to this operation is to prevent temporary warehouses by transferring semi-finished products to the next stage, and also if there is extra time, go to other idle machines. With this method, the number of required personnel was reduced from 18 people to 14 people, and instead, additional people were employed in other operations, such as assembly and quality control of semi-finished products.

In the next step, by adding an automatic machine that is equipped with a multi-stage washing operation system for cleaning - degreasing and phosphate of the parts, I was able to protect them from corrosion, which gives more paint adhesion and superior paint quality. Also, the fully automatic rose color unit with two paint spraying robots with the ability to paint static powder paint without manual intervention, which causes greater integrity and adhesion of the color in all parts of the product and decreasing total time span, as a result, we reached the required number of the production plan and the quality of the products increased.

[CE 1.9] Tasks Delegated and How accomplishing them

Due to the non-standard arrangement of the devices and the crowded space, I was assigned the task of reviewing the design of the layout of the production halls. The fact is that incorrect layout imposes uncalculated costs on the organization and in this factory due to the many bottlenecks in the production process, delays and idle times of machines and temporary warehouses and recent changes in the level of product demand and Future plans to change products, shows that the importance of this redesign has doubled. The main goal is to minimize material flow costs. In the production halls of the factory, it was observed that some machines that have similar functions and some work processes are in same sequence were arranged with a distance from each other.

For the current condition, only the static state was considered. For this purpose, a mathematical model was designed and taking into account parameters such as the location of the devices and the distance between them and the cost of the flow of items between both devices (including Area utilization cost and Material handling cost) and variable 0 and 1 means whether two devices are connected or not, I reached a general model and suggested to the factory manager to change the layout of the halls based on that. I used GAMS to calculate the algorithm.

By considering the above model, machines with similar functions were located in the same area.

[CE 1.10] Strategies Devised by You

The strategy that I had in mind and gave to the factory manager was the discussion of automating production processes. Regarding the transfer of goods and agile production, as well as the use of robots to perform operations such as assembly or moving products between devices. Although it will have significant costs for the initial implementation, it will save unforeseen and uncalculated costs in the future. Considering that the subject of my academic thesis was the use of robots in the production line, I presented a preliminary design for the assembly unit. It is also necessary to have an integrated software system in which all the processes of the organization, from purchasing and production to the management of personnel functions and the final sale, are recorded with great care. I offered to buy integrated software with the ability to customize according to my desired process.

[CE 1.11]

(i) Problem:

In order to increase productivity and better understanding in the organization, the lack of quality data regarding the process of material circulation from purchase to sale of the final product caused many problems and as a result, wrong decisions were made that ended up harming the organization.

Solution:

All the forms that existed for the circulation of materials and products in the organization were examined, from product purchase to product storage and production planning to final assembly and sales status and final invoice registration. In order to manage and improve productivity in the organization, the general information of this chain was needed, and there was no responsible department to record the information correctly. In the meeting I had with the factory manager, all the necessary forms had to be filled out with complete accuracy and precision. This cycle created a requirement in the entire organization and in a short time we saw valuable developments in various reports.

(ii) Problem:

Quality control of products (semi-finished products - purchased products - finished products) was not given serious attention, and accordingly, a lot of waste was created and there were no analytical reports for it.

Solution:

In periodic inspections, the purchased products were checked and every time it was found that some parts were missed in the purchased packages. Also, the amount of waste during production was significant. Also, in the stage after painting and final assembly, complete inspections were not performed. In this regard, I decided to divide the quality unit into two parts, quality control and quality assurance. Quality control is related to check the quality of semi-finished and purchased products, and for this procedure some forms were prepared to be completed, and people should record the data in them for future analytical measures, and this action should be done every working day. For cases where the waste during production was significant, production was stopped and technical meetings were held to check the condition of the production equipment and its maintenance and repairs.

For the final products, the quality assurance unit took on the task of checking the final products, and if the statistics of cases with technical defects were significant, the entire process was seriously investigated.

We tried to identify all the parameters that were necessary for quality control and quality assurance in order to determine the most waste cases and solve them radically.

[CE 1.12] How you worked with other team members

Although I was alone to implement this project, I had to establish continuous communication with the people involved in the production process for a better evaluation in order to move things forward, because all reforms should be done by them. Next, to check the final statistics, I had to interact with the sales and finance team to be able to have a complete assessment of the supply chain.

- Summary

[CE 1.13]

The result that I got from the implementation of this project was the importance of information and documents in increasing the productivity in a factory, and it is not possible without this two factors and lack of them imposes heavy costs on the organization. In short, it can be said that this project brought me valuable experiences to improve production processes in a factory and made me develop interesting ideas such as process automation - integrated software systems and supply chain management. It was very valuable that I was able to improve the performance of the production line by implementing the ideas and with the help of all the personnel - increase the production - increase the quality and finally the satisfaction of the employees and bring a new look to the development in the organization.

[CE 1.14]

During this time, my skills in working with excel increased and I created interesting and valuable reports with it. I also got acquainted with GAMS for model analysis, which was a valuable experience. I also used some statistical methods to analyze the amount of waste.